| Structural Element | Minimum Reinforcement | Maximum Reinforcement | Key IS Code Reference | Important Detailing Notes |
| --- | --- | --- | --- | --- |
| Beams (Tension Steel) | $\frac{A_s}{bd} = \frac{0.85}{f_y}$ (~0.17% for Fe 500) | 4.0% of gross area ($b \cdot D$) | IS 456:2000 Clause 26.5.1.1 | Max limit prevents steel congestion so concrete can flow smoothly around bars. |
| Beams (Compression) | No theoretical minimum (use 2 anchor bars) | 4.0% of gross area ($b \cdot D$) | IS 456:2000 Clause 26.5.1.2 | Total steel (Tension + Compression) should not practically exceed 8%. |
| Beams (Seismic / Ductile) | $\frac{0.24\sqrt{f_{ck}}}{f_y}$ (~0.24% for M25/Fe500) | 2.5% of gross area ($b \cdot D$) | IS 13920:2016 Clause 6.2.1 & 6.2.2 | The lower maximum limit (2.5%) ensures the beam fails in a ductile manner under heavy cyclic loads. |
| Columns | 0.8% of gross area ($A_g$) | 6.0% of gross area ($A_g$) (Practically restricted to 4.0%) | IS 456:2000 Clause 26.5.3.1 | The 4% practical limit applies at zones where longitudinal bars are lapped. |
| Slabs (HYSD bars) | 0.12% of gross area ($b \cdot D$) | Same as beams (4.0%) though rarely governed by it. | IS 456:2000 Clause 26.5.2.1 | Applicable for Fe 415 and Fe 500. (Increases to 0.15% if using Mild Steel). |
| Footings (Isolated/Combined) | 0.12% of gross area ($b \cdot D$) | Tied to beam/slab flexural criteria | IS 456:2000 Clause 34.5.2 | Refers back to the slab clause requirements for thick foundations. |
| Shear Walls (Vertical) | 0.25% of gross area | 4.0% of gross area | IS 13920:2016 Clause 10.1.6 & 10.1.9 | Minimum drops to 0.15% for thin wall configurations under light axial loads in IS 456. |
| Shear Walls (Horizontal) | 0.25% of gross area | N/A | IS 13920:2016 Clause 10.1.7 | Distributed uniformly across the wall height to combat diagonal shear cracking. |